Safety Device

Components

- Xiao esp32c3

- A push button

The block diagram

The circuit diagram:

Prototype 2

Components

Push button – 2

Xiao Esp32c3

Resister 10 k ohm -2

SIM800L

Working:

Code

#include <SoftwareSerial.h>

#include <TinyGPS++.h>

// GSM module

SoftwareSerial sim800(8, 9); // SIM800L RX, TX

// GPS module

SoftwareSerial gpsSerial(6, -1); // GPS TX → GPIO 6, RX unused

TinySPlus gps;

const int fireBtnPin = 2;

const int injuryBtnPin = 3;

bool fireSent = false;

bool injurySent = false;

String gpsLocation = "Waiting for GPS fix...";

const char* phoneNumber = "+1234567890"; // 🔁 Replace with your real number

void setup() {

pinMode(fireBtnPin, INPUT_PULLUP);

pinMode(injuryBtnPin, INPUT_PULLUP);

Serial.begin(115200);

sim800.begin(9600);

gpsSerial.begin(9600);

delay(1000);

sendAT("AT");

sendAT("AT+CMGF=1"); // Text SMS mode

}

void loop() {

updateGPS();

if (digitalRead(fireBtnPin) == LOW) {

delay(300); // Debounce

if (!fireSent) {

sendSMS("🔥 FIRE ALERT! Location: " + gpsLocation);

fireSent = true;

} else {

makeCall();

delay(10000);

hangUp();

}

}

if (digitalRead(injuryBtnPin) == LOW) {

delay(300); // Debounce

if (!injurySent) {

sendSMS("🩸 INJURY ALERT! Location: " + gpsLocation);

injurySent = true;

} else {

makeCall();

delay(10000);

hangUp();

}

}

}

void updateGPS() {

while (gpsSerial.available()) {

gps.encode(gpsSerial.read());

}

if (gps.location.isUpdated()) {

gpsLocation = "Lat: " + String(gps.location.lat(), 6) +

", Lon: " + String(gps.location.lng(), 6);

}

}

void sendSMS(String msg) {

sim800.println("AT+CMGS=\"" + String(phoneNumber) + "\"");

delay(1000);

sim800.print(msg);

delay(500);

sim800.write(26); // Ctrl+Z

Serial.println("SMS Sent: " + msg);

delay(3000);

}

void makeCall() {

sim800.println("ATD" + String(phoneNumber) + ";");

Serial.println("Calling " + String(phoneNumber));

}

void hangUp() {

sim800.println("ATH");

Serial.println("Call ended");

}

void sendAT(String cmd) {

sim800.println(cmd);

delay(500);

while (sim800.available()) {

Serial.write(sim800.read());

}

}